# Network Design and Implementation Using VLANs and Wireless Connectivity

## 1. Introduction

XYZ Company plans to establish a new branch office with three departments: Admin/IT, Finance/HR, and Customer Service/Reception. The objective of this project is to design and implement a network using one Cisco router and one Cisco switch while providing VLAN segmentation, wireless connectivity, automatic IP addressing, and inter-VLAN communication.

## 2. Objectives

- Design a small enterprise network.

- Create separate VLANs for each department.

- Configure wireless connectivity for users.

- Configure DHCP for automatic IP assignment.

- Enable communication between VLANs using Router-on-a-Stick.

## 3. Network Requirements

- One Cisco Router

- One Cisco Switch

- Three Departments

  - Admin/IT (VLAN 10)

  - Finance/HR (VLAN 20)

  - Customer Service/Reception (VLAN 30)

- Wireless Access Points

- DHCP Configuration

- Inter-VLAN Communication

## 4. Network Design

### VLAN Addressing Scheme

| Department | VLAN | Network Address | Gateway |
| --- | --- | --- | --- |
| Admin/IT | 10 | 192.168.1.0/26 | 192.168.1.1 |
| Finance/HR | 20 | 192.168.1.64/26 | 192.168.1.65 |
| Customer Service | 30 | 192.168.1.128/26 | 192.168.1.129 |

## 5. Implementation

### 5.1 VLAN Configuration

VLAN 10 was created for Admin/IT, VLAN 20 for Finance/HR, and VLAN 30 for Customer Service.

### 5.2 Router-on-a-Stick Configuration

Router subinterfaces were configured to enable communication between VLANs.

### 5.3 DHCP Configuration

DHCP pools were configured to automatically assign IPv4 addresses to all devices.

### 5.4 Wireless Configuration

Three wireless access points were configured:

- ADMIN_WIFI

- FINANCE_WIFI

- CUSTOMER_WIFI

Users connected through Wi-Fi and obtained IP addresses automatically.

## 6. Testing and Verification

The following tests were performed:

- VLAN Verification

- DHCP Verification

- Wireless Connectivity Verification

- Inter-VLAN Communication Verification

Successful ping results confirmed that devices in different VLANs could communicate through the router.

## 7. Results

The network was successfully implemented according to all requirements. VLAN segmentation improved network organization, DHCP simplified IP management, and wireless connectivity enabled user mobility. Inter-VLAN communication was achieved through Router-on-a-Stick configuration.

## 8. Conclusion

The project successfully implemented a secure and scalable network infrastructure using VLAN technology, DHCP services, wireless networking, and inter-VLAN routing. All stated requirements were achieved successfully.

Complete Network Topology

VLAN Configuration, Trunk Configuration

Router Configuration, DHCP Configuration

PC,  Smartphone Connected to WiFi, Laptop Connected to WiFi

Student Name: Mahesh Thilaksiri
Student ID: 32729
Module: Computer Networks
University: NSBM Green University